# Project Logbook: Smart Bike Lock – BLE Prototype

Students: Noah Zacharia, Peter Luster, Kirollos Melek

Project Title: Smart Bike Lock – Bluetooth Control Prototype

# Project Overview

The goal of this project is to design and prototype a smart bike lock controlled via a microcontroller (ESP32) over Bluetooth Low Energy (BLE).

The system allows a user to:

- Set a password to lock the bike

- Unlock the bike with the correct password

- Check the lock status

- Receive feedback via LED indicators

Initially, the project aimed to use RFID-based lock control, but after testing feasibility, the design pivoted to BLE due to reliability, ease of smartphone integration, and real-time feedback.

# Project Log Entries

## Entry 1

Initial Concept & Design Decisions | Week of Feb 1 -

- Idea - use RFID cards / tags to unlock a bike lock

- Challenges identified: iPhone cannot emulate arbitrary RFID tags easily; limited range for NFC/RFID

- Potentially explore other methods of unlocking, such as bluetooth

## Entry 2

Microcontroller Selection | Week of February 15 -

- Selected ESP32 DevKitC-1 initially for BLE support

- Issues encountered: Serial monitor did not work due to USB CDC limitations on S3 board. - Workaround: Proceed with LED feedback testing and BLE notifications; use BLE to simulate debugging messages.

## Entry 3

BLE Architecture | Week of March 1

- Created BLE server on ESP32 with:

  - Service UUID for the bike lock

  - Command characteristic (write-only) for lock/unlock/set password

  - Status characteristic (read/notify) to report lock state and debug messages

  - Implemented password-based authentication via BLE commands

  - LED indicators: Red = locked, Yellow = unlocked.

## Entry 4

Command Structure & Testing | Week of March 1

- Designed BLE command syntax for mobile app: SET 1234, UNLOCK 1234, LOCK, STATUS.

- Tested with LightBlue app

- Verified LEDs changed correctly based on command and password validation

- Learned JSON commands were too cumbersome on mobile devices; simplified syntax improves usability.

## Entry 5

Serial Debugging Challenges | Week of March 1

- Attempted Serial.println() for debugging

- On ESP32-S3: no serial output visible in PlatformIO

- We will probably have to use a different dev board, but that is fine since the one we chose is a little large